NSTX

Stress Analysis of ATJ Center Stack Tiles and Fasteners

NSTXU-CALC-11-03-00

May 9, 2011

Prepared By:

___________________________________

Art Brooks, Engineering Analyst

Reviewed By:

________________________________________

????

Approved By:

_______________________________________

Kelsey Tresemer, Cognizant Engineer

PPPL Calculation Form

Calculation # 	NSTXU-CALC-11-03-00     Revision #00     WP #, if any  1707

(ENG-032)

Purpose of Calculation: (Define why the calculation is being performed.)

Stress Analysis of the ATJ Tiles and Supports

References (List any source of design information including computer program titles and revision levels.)

See attached report

Assumptions (Identify all assumptions made as part of this calculation.)

See attached report

Calculation (Calculation is either documented here or attached)

See attached report

Conclusion (Specify whether or not the purpose of the calculation was accomplished.)

The Center Stack Tiles for the NSTX-CSU program are shown to be capable of withstanding the original GRD heat flux requirements using the prescribed ATJ graphite. The tile mounting scheme, consisting of T-bar supports for the CS Angle Section (CSAS) Tiles and the Inboard Divertor Horizontal (IBDhs) and Vertical (IBDvs) Tiles, and the tray support for the Center Stack First Wall (CSFW) Tiles is adequate to support the tiles against the anticipated thermal, eddy current and halo current loads with acceptable bolt loads.

Cognizant Engineer’s printed name, signature, and date

Kelsey Tresemer

I have reviewed this calculation and, to my professional satisfaction, it is properly performed and correct.

Checker’s printed name, signature, and date

________________________________________________________________________

Executive Summary

The Center Stack Tiles for the NSTX-CSU program are shown to be capable of withstanding the original GRD heat flux requirements using the prescribed ATJ graphite. The tile mounting scheme, consisting of T-bar supports for the CS Angle Section (CSAS) Tiles and the Inboard Divertor Horizontal (IBDhs) and Vertical (IBDvs) Tiles, and the tray support for the Center Stack First Wall (CSFW) Tiles is adequate to support the tiles against the anticipated thermal, eddy current and halo current loads with acceptable bolt loads.

This is premised on the poloidal flowing halo current’s interaction with the TF field always results in tile forces which are away from the plasma, regardless of the plasma current and TF field directions as observed in NSTX operation. While the interaction of toroidal flowing halo currents, which will be in both directions due to the Toroidal Peaking, with the PF field produce forces both toward and away from the plasma, they are shown to be small relative to the poloidal current forces and result in net forces away from the plasma. If net forces were reversed, halo currents from a 2 MA plasma may not be tolerable due to high tensile stresses in the ATJ.

The analysis shows that the inclusion of Grafoil under the CSAS, IBDvs and IBDhs combined with the active cooling will significantly limit the thermal ratcheting of the tiles whether Li coated (with assumed emissivity of 0.3) or uncoated (with assumed emissivity of 0.7). The active cooling also offers adequate protection of the neighboring PF and OH coils and reduces the heating of the CS Casing. The flow rate and back pressure are high enough to avoid boiling of the water.

The Grafoil is shown to be structural compliant to allow relatively free thermal expansion of the tiles provided the bolts are only lightly preloaded and do not over compress the Grafoil.

Tile Thermal and EM Stresses are within acceptable limits for ATJ graphite.

Introduction

The Center Stack Casing (CSC) Plasma Facing Components (PFC) tiles are designed to protect the Center Stack from the high heat fluxes of the plasma. They are divided into four sections of tiles referred to in the General Requirements Document (GRD) as the Inboard Divertor Horizontal (IBDhs) and Vertical (IBDvs) Tiles, the CS Angle Section (CSAS) Tiles, and the Center Stack First Wall (CSFW) Tiles. The GRD requires all CSC PFC tiles be designed using high-grade graphite material. The use of carbon fiber composites is not permitted due to Lithium retention of the coarse weave. The available tile thickness is also dictated by the GRD. As a result the goal of the analysis is to establish safe operating limits up to the GRD desired level. Tile mounting details have been optimized within these constraints to enhance the thermal performance while withstanding the electromagnetic loading from plasma disruption induced eddy currents and halo currents.

Heat is removed from the CSAS, IBDhs and IBDvs tiles by radiation to cooled outboard components (OD, PP & VV) and by the CSC water cooling system. The CSFW tiles are only radiation cooled since the CSC cooling does not extend up between the Casing and the OH coils. One of the design decisions resulting from this analysis is the use of a thermal interface material – Grafoil – between the tiles and the CSC. The original plan was to limit the heat transfer between the tiles and the CSC by not using Grafoil and relying on radiation only, out of concern about over heating the water.  There are now four CSC cooling circuits in the design (two on top and two on the bottom) where there are dedicated circuits for the high heat flux IBDhs. Analysis has shown them to be adequate to safely remove the heat during the transient. The result is the water cooled tiles do not thermally ratchet with repeated pulsing. There will be ratcheting of the uncooled CSFW but the incident heat fluxes are low as would be the peak temperatures.

Assumptions

The tile mounting schemes are designed to permit relatively free thermal expansion, minimizing thermal stresses. The CSAS, IBDhs and IBDvs tiles use T-bar supports held by bolts with Belleville washers and with compliant Grafoil underneath. The bolts are lightly loaded (500 N or 112 lbs) to permit bowing of the tiles under thermal gradients. Tolerances are set to assure the load path for EM forces is directly into the Grafoil and not the bending the tile over the T-Bar.

The analysis assumes the poloidal flowing halo current’s interaction with the TF field always results in tile forces which are away from the plasma, regardless of the plasma current and TF field directions as observed in NSTX operation. While the interaction of toroidal flowing halo currents, which will be in both directions due to the Toroidal Peaking, with the PF field produce forces both toward and away from the plasma, they are shown to be small relative to the poloidal current forces and result in net forces away from the plasma. If net forces were reversed, halo currents from a 2 MA plasma may not be tolerable due to high tensile stresses in the ATJ.

The analysis is done using the average heat fluxes associated with a 14 MW plasma of 5 second duration pulse with 1200 second rep rate.

Method of Analysis

ANSYS models were used to analyze the thermal and structural response of each of the four tile types. ProE models of the tile and supports were imported into ANSYS Classic. A thermal transient was run to generate the temperature distribution on the ATJ tiles.

Eddy currents were calculated using max values of dB/dt (vertical and radial) at the tile locations found from scanning the 5 disruption scenarios given in Table 2.2 of the GRD.  The scans were done using the SPARK code with previously generated models of the VV, CS and PP.

The background maximum field values were obtained by scanning thru the 96 operating scenarios specified in the Design Point Spreadsheet “NSTX_CS_Upgrade_100504.xls” using a FORTRAN code built on the Magnetics Library routine FICOI. This was found to be in agreement with results generated by others using the OPERA code.

Halo currents in the tiles are based on the resistive sharing of poloidal currents with the CSC. While the tiles themselves are not poloidally continuous, it is postulated that during a halo current strike plasma fills the gaps between the participating tiles and shorts them out. At an estimate temperature of 10ev, plasma resistivity is comparable to ATJ graphite.

The tile thermal and structural performance is based on the use of ATJ graphite who’s properties are given below.

Results

A 1-D thermal performance of ATJ Graphite was generated at heat fluxes varying from 5 15 MW/m2 (DN) to 15 MW/m2 (SN) for comparison. It suggests that the design which is governed per the GRD by the DN operation for 5 sec would limit single null operation to under 1 sec.

A 2-D axisymetric thermal model to the previously run was modified to reflect the use of Grafoil under the tiles. The model was also modified to include the effect of water transport (using ANSYS fluid116 elements) instead of just using an effective film coefficient as used in earlier analyses. This limited the thermal ratcheting while still providing adequate limits on the water temperature rise as shown below.

A halo current distribution model was also created to investigate the direction of forces on the tile. This was crucial to the structural performance. Results show that forces are always away from the plasma and into the supporting CSC which limits the tile stresses since the tile is effective supported off it base and not the thin sections at the T-Bar.

As a result of Toroidal Peak, there is a resistive redistribution of current primary in the low resistance section of the IBDhs. When crossed with the radial PF this will cause some tiles to experience forces into the wall and others away from the wall. The IBDhs current toroidal distribution is driven more by the TPF than by the assumed toroidal angle between strike in and out. Peak toroidal current in IBDhs is 27.3 kA of which 4.9 kA flows thru the ATJ tile assuming a resistive distribution between tile and casing.

Results for Individual Tiles:

IBDhs

IBDvs

CSAS

CSFW

Summary

The tables below summarize the peak temperatures and stresses from the analysis for the given heat load:

The Center Stack Tiles for the NSTX-CSU program are shown to be capable of withstanding the original GRD heat flux requirements using the prescribed ATJ graphite with Tensile Strength of 26 MPa and Compressive Strength of 66 MPa. Results are based on average Tile surface heating. The IBDhs shows a hot spot at the corner of the tile closest to the X-point due to assumed heating from both faces which may be (or may not be) conservative.

The tile mounting scheme, consisting of T-bar supports for the CS Angle Section (CSAS) Tiles and the Inboard Divertor Horizontal (IBDhs) and Vertical (IBDvs) Tiles, and the tray support for the Center Stack First Wall (CSFW) Tiles is adequate to support the tiles against the anticipated thermal, eddy current and halo current loads with acceptable bolt loads. The load paths are such as to dump the net tile forces from Halo and Eddy Currents directly into the CSC. The supports offer flexible constraint on the tile thermal expansion without carrying significant load.

To repeat what was said earlier, the EM load direction is premised on the poloidal flowing halo current’s interaction with the TF field always results in tile forces which are away from the plasma, regardless of the plasma current and TF field directions as observed in NSTX operation. While the interaction of toroidal flowing halo currents, which will be in both directions due to the Toroidal Peaking, with the PF field produce forces both toward and away from the plasma, they are shown to be small relative to the poloidal current forces and result in net forces away from the plasma. If net forces were reversed, halo currents from a 2 MA plasma may not be tolerable due to high tensile stresses in the ATJ.

The analysis shows that the inclusion of Grafoil under the CSAS, IBDvs and IBDhs combined with the active cooling will significantly limit the thermal ratcheting of the tiles whether Li coated (with assumed emissivity of 0.3) or uncoated (with assumed emissivity of 0.7). The active cooling also offers adequate protection of the neighboring

PF and OH coils and reduces the heating of the CS Casing. The flow rate and back pressure are high enough to avoid boiling of the water.

The Grafoil is shown to be structural compliant to allow relatively free thermal expansion of the tiles provided the bolts are only lightly preloaded and do not over compress the Grafoil.

IBhs and IBDvs (top) and CSAS and CSFW (bot) Thermal and EM Stresses are within acceptable limits for ATJ graphite.

References

- NSTX_CSU-RQMTS-GRD General Requirements Documents, Rev 3

- Design Point Spreadsheet “NSTX_CS_Upgrade_100504.xls”

- NSTXU-Calc-11-01-00 Global Thermal Analysis of Center Stack Heat Balance, Dated February 15, 2011

- ProE Model of Center Stack Tiles - aj_center_case_analysis_rev2.asm

- Spreadsheet of Disruption Data - Disruption_scenario_currents_v2.xlsx, by Jon Menard, received 7/2/2010

- Discussions with Stefan Gerhardt  on modeling of halo currents for NSTX