NSTX

Global Model – Model Description, Mesh Generation, Results

NSTXU-CALC-10-01-02

Rev 2

October 29, 2010

Prepared By:

___________________________________

Peter Titus, PPPL Mechanical Engineering

Approved By:

_______________________________________

Phil Heitzenroeder, Head, Mechanical Engineering

PPPL Calculation Form

Calculation # 	NSTXU-CALC-10-01	Revision #  02		WP #, if any

(ENG-032)

Purpose of Calculation: (Define why the calculation is being performed.)

References (List any source of design information including computer program titles and revision levels.)

Assumptions (Identify all assumptions made as part of this calculation.)

Calculation (Calculation is either documented here or attached)

Conclusion (Specify whether or not the purpose of the calculation was accomplished.)

Cognizant Engineer’s printed name, signature, and date

I have reviewed this calculation and, to my professional satisfaction, it is properly performed and correct.

Checker’s printed name, signature, and date

Record of Changes

| Date | Revision | Description of Change |
| --- | --- | --- |
| 8/13/2009 | 0 | Initial Issue as NSTX-CALC-13-01-00 |
| 8/27/2009 | 1 | Updated calculation |
| 10/29/2010 | 2 | Revised calculation number to NSTX-CALC-10-02-00 since this covers the Entire torus System. Also added Record of Changes and references to Figures and Tables. |

# Table of Contents

Section.Paragraph

Executive Summary								1.0

References									2.0

Model										3.0

Modeling Elements & Real Constraints					3.1

Input Currents								3.2

Peak Fields								3.3

Mesh Generation and Model Creation					3.4

Global Model History							3.5

Run Log									3.6

Materials and Allowables							4.0

Allowables								4.1

Results									5.0

Upper Flex Plate/Diaphragm (Replaces the Gear Tooth Connection)		5.1

TF Joint “Loop” Vertical Field						5.2

TF Outer Leg Bending  							5.3

TF Outer Leg Bond Shear 							5.4

Toroidal Displacements at TF Flex Joint					5.5

Center Stack – TF Inner Leg Torsional Shear 				5.6

Outer PF Support Structure 						5.7

Main Support Column to Vessel Connection 				5.8

Centerstack Torsional Displacement at OH Bellevilles 			5.9

Net Load on Centerstack 							5.10

Vessel Stresses								5.11

Center Stack Casing Thermal Stress 					5.12

Bake-Out 								5.13

# 1.0 Executive Summary

The Global model of NSTX Center Stack Upgrade (NSTX-CSU)  provides a simulation of the overall behavior of the machine. It provides boundary conditions for local models and sub Models , or allows inclusion of the detailed models of components in the global model.  In many cases it has been built from  from other available model segments – The upper and lower head sections of the vessel model come from H.M. Fan’s early vessel models. The cylindrical shell that contains the mid plane ports comes from a vessel model built by Srinivasa Avasarala from the Pro–E model of the vessel.  In some instances parts of the global model were exported to be evaluateds in more detail. Multiple scenarios from the NSTX design point  are run using the global model. The design points are publised on the web and are maintained by C. Neumeyer. As of this issue of the calculation, 70 of the 90 normal operating current sets published in the July 2009 design point have been run in the global model. The September 8 design point has a revision to the OH current variations and these have not yet been run.  Loads from  normal operating current sets are in general much less severe than loads that are based on worst case power supply currents.  In order to compare the global model results with some of the local models that have been run, some of the “worst case” currents have been run in the global model. The outer TF reinforcements are an example of this. Results reported in sub paragraphs of section 8 have been used to qualify components, check results and guide the need for further analyses. The outer TF leg reinforcements discussed in  section 8.3 and in NSTX calculation number 132-04-00 are based on two pairs of current sets. These are intended to maximize the out-of-plane loading on the TF outer legs for an up-down symmetric loading and an up-down asymmetric loading that causes large net torques on the outer legs. These two current sets were included in the loading analyzed in the global model. Behavior of the two analyses is consistent.  Section 8.3 discusses these results and adds a qualifiucationn of the bending related bond shear in the TF outer leg. Section 8.1 documents the acceptable stresses in the diaphram plate that replaces the gear tooth torsional connection between the centerstack and the outer umbrella structure.Section 8.5 provided global displacements to the detailed analysis of the flex joint [7]  Section 8.6 is to date, the only treatment that shows acceptability of the torsional shear in the inner leg. Section 8.9 similarly profided guidance on global twist in the evaluation of the centerstack OH support details. Section 8.8 shows the stresses and loading around the I beam column attachmeents to the vessel and points to the need to evaluate the weld details of this connection.

Figure 1.0-1 Global Model Status as of June 22 2009

# 2.0 References

[1]  http://www.pppl.gov/~neumeyer/NSTX_CSU/Design_Point.html  Dated 2 -17- 2009

[2] Fusion Ignition Research Experiment Structural Design Criteria; Doc. No.  11_FIRE_-DesCrit_IZ_022499.doc; February, 1999

[3] "MHD and Fusion Magnets, Field and Force Design Concepts", R.J.Thome, John Tarrh, Wiley Interscience, 1982

# [4] “Analysis of TF Outer Leg ” Han Zhang  NSTX Calculation Number 132-04-00

[5] “Estimated and Compiled Properties of Glass/101K Epoxy/Kapton Composite Properties at Room Temperature” Report to Jim Chrzanowski Princeton Plasma Physics Laboratory July 15, 2009 R. P. Reed Cryogenic Materials, Inc.Boulder, CO

[6] NSTX Structural Design Criteria Document, I. Zatz

[7] “TF Flex Joint and TF Bundle Stub” T. Willard, NSTX-CALC-132-06-00

[8] “Influence Coefficients”, R. Hatcher, NSTX-CALC-13-03-00

[9] NSTX Design Point Sep 8 2009  http://www.pppl.gov/~neumeyer/NSTX_CSU/Design_Point.html

[10] Maximum and Minimum Loads on the NSTX OH and PF Coils, and Coil Groupings” P.Titus   NSTX-CALC-13-02-00

[11]  ANSYS Structural Analysis Program, Revision 10.0  Swanson Analysis Systems

[12] "Mechanical, Electrical and Thermal Characterization of G10CR  and G11CR Glass Cloth/Epoxy Laminates Between Room Temperature and 4 deg. K", M.B. Kasen et al , National Bureau of Standards, Boulder Colorado.

[12] National Spherical Torus Experiment NSTX CENTER STACK UPGRADE GENERAL REQUIREMENTS DOCUMENT NSTX_CSU-RQMTS-GRD Revision 0 March 30, 2009 Prepared By: Charles Neumeyer NSTX Project Engineering Manager

# Model

Figure 3.0-1 Global Model Status as of June 22 2009

The Global model of NSTX Center Stack Upgrade (NSTX-CSU)  provides a simulation of the overall behavior of the machine. It provides boundary conditions for local models and sub Models , or allows inclusion of the detailed models of components in the global model.  In many cases it has been built from  from other available model segments – The upper and lower head sections of the vessel model come from H.M. Fan’s early vessel models. The cylindrical shell that contains the mid plane ports comes from a vessel model built by Srinivasa Avasarala from the Pro–E model of the vessel.  Thermal Extremes, bake-out and operating temperatures in the centerstack casing are included as separate load steps . In another load step vacuum loads are applied. In some runs these are left on and in others they are turned off. To get the proper load balance, all the port openinngs must be closed and properly loaded. At this writing there are still some vessel shell areas that are reversed and some port openings that are not closed.

Figure 3.0-2 Global Model as of October 2009

## 3.1 Modeling Elements, Real Constants

Figure 3.1-2 TF real constants

Figure 3.1-3 View Inside Umbrella Structure

Figure 3.1-1 PF Coil Real Constants

TF Outer Leg Dimensions

Figure 3.1-4 Single Turn Dimensions (The Outboard Leg has 3 turns

Figure 3.1-5 TF Outer Leg Dimensions

Figure 3.1-6

Coil Builds

#33 is the Plasma

| # | r | z | dr | dz | nx | nz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | .2344 | .0021 | .01 | 4.3419 | 2 | 20 |
| 2 | .2461 | .0067 | .01 | 4.2803 | 2 | 20 |
| 3 | .2577 | .0022 | .01 | 4.2538 | 2 | 20 |
| 4 | .2693 | -.0021 | .01 | 4.1745 | 2 | 20 |
| 5 | .3239 | 1.5906 | .0413 | .3265 | 4 | 4 |
| 6 | .4142 | 1.8252 | .042 | .1206 | 4 | 4 |
| 7 | .56 | 1.8252 | .042 | .1206 | 4 | 4 |
| 8 | .7992 | 1.8526 | .1627 | .068 | 4 | 4 |
| 9 | .7992 | 1.9335 | .1627 | .068 | 4 | 4 |
| 10 | 1.4829 | 1.5696 | .1631 | .034 | 4 | 4 |
| 11 | 1.4945 | 1.5356 | .1864 | .034 | 4 | 4 |
| 12 | 1.4829 | 1.6505 | .1631 | .034 | 4 | 4 |
| 13 | 1.4945 | 1.6165 | .1864 | .034 | 4 | 4 |
| 14 | 1.795 | .8711 | .0922 | .034 | 4 | 4 |
| 15 | 1.8065 | .9051 | .1153 | .034 | 4 | 4 |
| 16 | 1.7946 | .8072 | .0915 | .068 | 4 | 4 |
| 17 | 1.795 | -.8711 | .0922 | .034 | 4 | 4 |
| 18 | 1.8065 | -.9051 | .1153 | .034 | 4 | 4 |
| 19 | 1.7946 | -.8072 | .0915 | .068 | 4 | 4 |
| 20 | 2.0118 | .6489 | .1359 | .0685 | 4 | 4 |
| 21 | 2.0118 | .5751 | .1359 | .0685 | 4 | 4 |
| 22 | 2.0118 | -.6489 | .1359 | .0685 | 4 | 4 |
| 23 | 2.0118 | -.5751 | .1359 | .0685 | 4 | 4 |
| 24 | 1.4829 | -1.5696 | .1631 | .034 | 4 | 4 |
| 25 | 1.4945 | -1.5356 | .1864 | .034 | 4 | 4 |
| 26 | 1.4829 | -1.6505 | .1631 | .034 | 4 | 4 |
| 27 | 1.4945 | -1.6165 | .1864 | .034 | 4 | 4 |
| 28 | .7992 | -1.8526 | .1627 | .068 | 4 | 4 |
| 29 | .7992 | -1.9335 | .1627 | .068 | 4 | 4 |
| 30 | .56 | -1.8252 | .042 | .1206 | 4 | 4 |
| 31 | .4142 | -1.8252 | .042 | .1206 | 4 | 4 |
| 32 | .3239 | -1.5906 | .0413 | .3265 | 4 | 4 |
| 33 | .9344 | 0 | .5696 | 1 | 6 | 8 |

Figure 3.1-9 Details of the PF3,4,5,U&L Support

Figure 3.1-7

Figure 3.1-8 Details of the Tangential Radius Rod and PF 3,4,5,u&L Support

Figure 3.1-8 Details of the Tangential Radius Rod and PF 3,4,5,u&L Support

## 3.2 Input Currents

The most recent analyses are based on the current sets included in the design point:

http://www.pppl.gov/~neumeyer/NSTX_CSU/Design_Point.html

In addition, some earlier runs used a series of equilibria from Jon Menard and worst case currents provided by C. Neumeyer. These are shown below:

PF Scenario Currents In Mat – (Prior to 90 Design Point Scenarios)

| Coil # | TFON | IM | -0.1 | -0.05 | 0 | 0.05 | 0.1 | Worst 1 | Worst 2 | Worst3 | Worst4 | Worst5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
|  | Nst1 | Nst2 | Nst3 | Nst4 | Nst5 | Nst6 | Nst7 | Nsw3 | Nsw4 | Nsw5 | Nsw6 | Nsw7 |
| 1 | 0 | 5.88 | .000 | .000 | .000 | .000 | .000 | -5.88 | 5.88 | 5.88 | -1.47 | -1.47 |
| 2 | 0 | 5.808 | .000 | .000 | .000 | .000 | .000 | -5.808 | 5.808 | 5.808 | -5.808 | -1.452 |
| 3 | 0 | 5.76 | .000 | .000 | .000 | .000 | .000 | -5.76 | 5.76 | 5.76 | -5.76 | -1.92 |
| 4 | 0 | 5.664 | .000 | .000 | .000 | .000 | .000 | -5.664 | 5.664 | 5.664 | -5.664 | -1.416 |
| 5 | 0 | 0 | 7.172 | 7.196 | 7.234 | 7.348 | 7.452 | 0.784 | 0.784 | 0.784 | 0.784 | 0.784 |
| 6 | 0 | 0 | -5.650 | -4.763 | -3.628 | -2.331 | -.946 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 |
| 7 | 0 | 0 | -4.922 | -4.014 | -2.936 | -1.755 | -.517 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| 8 | 0 | 0 | 4.484 | 4.307 | 3.941 | 3.401 | 2.772 | 0.168 | 0.168 | 0.168 | 0.168 | 0.168 |
| 9 | 0 | 0 | 4.484 | 4.307 | 3.941 | 3.401 | 2.772 | 0.168 | 0.168 | 0.168 | 0.168 | 0.168 |
| 10 | 0 | 0 | -1.058 | -1.426 | -1.655 | -1.720 | -1.690 | -0.112 | -0.112 | -0.112 | -0.112 | -0.112 |
| 11 | 0 | 0 | -1.058 | -1.426 | -1.655 | -1.720 | -1.690 | -0.128 | -0.128 | -0.128 | -0.128 | -0.128 |
| 12 | 0 | 0 | -1.058 | -1.426 | -1.655 | -1.720 | -1.690 | -0.112 | -0.112 | -0.112 | -0.112 | -0.112 |
| 13 | 0 | 0 | -1.058 | -1.426 | -1.655 | -1.720 | -1.690 | -0.128 | -0.128 | -0.128 | -0.128 | -0.128 |
| 14 | 0 | 0 | -2.388 | -1.183 | -.206 | .488 | .923 | -0.08 | -0.08 | -0.08 | -0.08 | -0.08 |
| 15 | 0 | 0 | -2.388 | -1.183 | -.206 | .488 | .923 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 |
| 16 | 0 | 0 | -2.388 | -1.183 | -.206 | .488 | .923 | -0.16 | -0.16 | -0.16 | -0.16 | -0.16 |
| 17 | 0 | 0 | -2.388 | -1.183 | -.206 | .488 | .923 | -0.08 | -0.08 | -0.08 | -0.08 | -0.08 |
| 18 | 0 | 0 | -2.388 | -1.183 | -.206 | .488 | .923 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 |
| 19 | 0 | 0 | -2.388 | -1.183 | -.206 | .488 | .923 | -0.16 | -0.16 | -0.16 | -0.16 | -0.16 |
| 20 | 0 | 0 | -3.374 | -4.340 | -5.139 | -5.771 | -6.210 | -0.384 | -0.384 | -0.384 | -0.384 | -0.384 |
| 21 | 0 | 0 | -3.374 | -4.340 | -5.139 | -5.771 | -6.210 | -0.384 | -0.384 | -0.384 | -0.384 | -0.384 |
| 22 | 0 | 0 | -3.374 | -4.340 | -5.139 | -5.771 | -6.210 | -0.384 | -0.384 | -0.384 | -0.384 | -0.384 |
| 23 | 0 | 0 | -3.374 | -4.340 | -5.139 | -5.771 | -6.210 | -0.384 | -0.384 | -0.384 | -0.384 | -0.384 |
| 24 | 0 | 0 | -1.058 | -1.426 | -1.655 | -1.720 | -1.690 | -0.112 | -0.112 | -0.112 | -0.112 | -0.112 |
| 25 | 0 | 0 | -1.058 | -1.426 | -1.655 | -1.720 | -1.690 | -0.128 | -0.128 | -0.128 | -0.128 | -0.128 |
| 26 | 0 | 0 | -1.058 | -1.426 | -1.655 | -1.720 | -1.690 | -0.112 | -0.112 | -0.112 | -0.112 | -0.112 |
| 27 | 0 | 0 | -1.058 | -1.426 | -1.655 | -1.720 | -1.690 | -0.128 | -0.032 | -0.128 | -0.128 | -0.128 |
| 28 | 0 | 0 | 4.484 | 4.307 | 3.941 | 3.401 | 2.772 | 0.168 | 0.168 | 0.168 | 0.168 | 0.168 |
| 29 | 0 | 0 | 4.484 | 4.307 | 3.941 | 3.401 | 2.772 | 0.168 | 0.168 | 0.168 | 0.168 | 0.168 |
| 30 | 0 | 0 | -4.922 | -4.014 | -2.936 | -1.755 | -.517 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| 31 | 0 | 0 | -5.650 | -4.763 | -3.628 | -2.331 | -.946 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 |
| 32 | 0 | 0 | 7.172 | 7.196 | 7.234 | 7.348 | 7.452 | 0.784 | 0.784 | 0.784 | 0.784 | 0.784 |
| 33 | 0 | 0 | 2.000 | 2.000 | 2.000 | 2.000 | 2.000 | 2 | 2 | 2 | 2 | 2 |

## Table 3.2-1 Equilibria TablesFields

## 3.3 Fields and Forces

The peak field from the load files used in the global model is 4.9T. The peak field from the electromagnetic current diffusion model is 4.2T. They used different TF inner leg dimensions from different design point published throughout 2009.

Figure 3.3-1 Peak Fields

Figure 3.3-2 Peak Fields

## 3.4 Mesh Generation and Model Creation.

The mesh generation and calculation of the Lorentz forces is done outside of ANSYS using a code written by the author of this report. It is the same code that was used by the author in the “first” EDA. It is a Biot Savart solution based on single stick field calculations from Dick Thomes book [3] with some help from Pillsbury’s FIELD3D code

Figure 3.4-2 TFON Loading

Figure 3.4-1  H.M. Fan’s Quarter Symmetry Model of the Vessel and Passive Plates

to catch all the coincident current vectors, and other singularities. The analysts in the first ITER EDA went through an exercise to compare loads calculated by the US, RF and by Cees Jong in ANSYS, and that the US analyses were “OK”. Agreement was not good on net loads on coils that should net to zero – all the methods had some residuals, but summations on coil segments agreed very well. code Some information on the code, named FTM (Win98) and NTFTM2 (NT,XP),  is available at: http://198.125.178.188/ftm/manual.pdf  ).

The loads can be calculated  within ANSYS, but the constraints on magnetic modeling vs. structural modeling make it tough to vary the structure. Coil mesh files and load files are separate in the structural model, and  the support structure can be changed without changing the magnetics. The model segments and load files are input with the /INPUT command within the ANSYS batch file and look like ANSYS text commands. All the solid elements are SOLID 45’s. Higher order elements are not used because the force calculations, if done outside of ANSYS, need some correction at the mid side nodes. Gap elements are the point-to-point type – Type 52. The use of the authors meshing tools is largely a result of wanting to control the mesh alignment at the interfaces, required by the point to point elements. Target surface elements take up too much solution time.

## Global Model History

Figure 3.5-1 Jacking ring concept

The global model has evolved through the Conceptual design activity. Early models were used to address alternate joint concepts. Variations in the outer leg support modifications were also considered. The TF outer leg support truss was  modeled in the global model, and shown in Figure 4.0-1. Only the tangential radius rod results are reported in this calculation.

Figure 3.5-2 A Global Model of a Joint concept that was close to Bob Woolley’s Original Concept

## Run Log

# Global Model Runs

| Run File | Date | Mandrel File | Coil File | Load Files | Comments |
| --- | --- | --- | --- | --- | --- |
| Nstx06.txt | 2-2009 |  |  |  |  |
| Nstx17.txt | 9-2009 |  |  |  | Linear (Links Replace Gaps) |
| Nstx18.txt | 9-2009 |  |  |  | Non-Linear Gaps |

Figure 3.6-1 Model used in Run#6

# 4.0 Materials and Allowables

4.1 Allowables

Design guidance and structural criterial are contained in the NSTX structural design criterial [6]

4.1-1 Copper Conductor Allowable:

The TF copper ultimate is 39,000 psi or 270 MPa . The yield is 38ksi (262 MPa).  Sm is 2/3 yield or 25.3ksi or 173 MPa – for adequate ductility, which is the case with this copper which has a minimum of 24% elongation.  Note that the ½ ultimate is not invoked for the conductor (It is for other structural materials) . These stresses should be further reduced to consider the effects of operation at 100C. This effect is estimated to be 10% so the Sm value is 156 MPa.

- From: I-4.1.1   Design Tresca Stress Values (Sm), NSTX_DesCrit_IZ_080103.doc

- (a) For conventional (i.e., non-superconducting) conductor materials, the design Tresca stress values (Sm) shall be 2/3 of the specified minimum yield strength at temperature, for materials where sufficient ductility is demonstrated (see Section I-4.1.2). *

- It is expected that the CS would be a similar hardness to the TF so that it could be wound readily. For the stress gradient in a solenoid, the bending allowable is used. The bending allowable is 1.5*156 or 233MPa

4.2-2  Stainless Steel Allowable

| Material | Sm | 1.5Sm |
| --- | --- | --- |
| 316 LN SST | 183Mpa (26.6 ksi) | 275Mpa (40ksi) |
| 316 LN SST weld | 160MPa(23.2ksi) | 241MPa(35ksi) |
| 304 Vessel |  | 45 ksi |

From Dick Reed Reports/Conversations [5] :

Shear strength, short-beam-shear, interlaminar

Without Kapton		              65 MPa    (TF, PF1 a,b,c)

With Kapton		       		 40 MPa (CS)

Estimated Strength at Copper Bond   65 MPa/2 =32.5 MPa (All Coils)

From Criteria Document:

I-5.2.1.3  Shear Stress Allowable

The shear-stress allowable, Ss, for an insulating material is most strongly a function of the particular material and processing method chosen, the loading conditions, the temperature, and the radiation exposure level.  The shear strength of insulating materials depends strongly on the applied compressive stress.  Therefore, the following conditions must be met for either static or fatigue conditions:

Ss =	[2/3 to ]+ [c2 x Sc(n)]

2/3 of 32.5 MPa = 21.7 MPa

5.0 Results

5.1 Upper Flex Plate/Diaphragm (Replaces the Gear Tooth Connection)

Hot Central Column, Cold Vessel

Central Column Expands 9mm

5/8” Flex/Diaphram, 150 MPa

Note Non-Uniform Stress when TF Expands

- Vessel at 150C during Bake-Out RT Central Column

- Vessel Expands +8mm

Flex/Diaphram Stress is 135 MPa

Note Uniform Stress at Edge

5.2 TF Joint “Loop” Vertical Field

Normal Operation Currents Produce <.1T

This is the Loading used in Fatigue Calculations

Ron Hatcher Gave us the Worst Vertical Field , .3T,  to Design the Strap to.

The local model of the flex joint developed by Tom Willard  [6] employs an approximation to the poloidal field. This postprocess of the load files used in the global model shows that the max poloidal or vertical field provided by R. Hatcher, corresponds to an extreme case. Normal operating currents produce fields closer to .1T. The flex needs to be designed foer a worst case, but fatigue evaluations should be based on the much more likely .1T vertical field. 
5.3 TF Outer Leg Bending

Figure 5.3-1 TF Outer Leg Bend Stress

Han Zhang has addressed the need for outer leg reinforcement in her calculation number NSTX CALC 132-04-00. This includes an evolutions in design concepts intended to support the outer TF legs for in-plane as well as out of plane loads. An early truss concept was eliminated from the running because of interference problems with many diagnostics and waveguides installed in the bays. The tangential radius rud restraint concept is the present design. The truss was  modeled in the global model, and shown in Figure 4.0-1. Only the tangential radius rod results are reported in this calculation.

Corresponds to Han’s Outer Leg with “Worst Case”  Currents Provided by C. Neumeyer

Figure 5.3-2 Global Model TF Outer Leg Bending Stress

Figure 5.3-3The Toroidal Width of the TF Outer Leg Should Be 6 inches. Stresses would scale as the section modulus or by d^3

Figure 5.3-4 TF Outer Leg Bend Stress from [4]

The Global model contains an error that over-estimates the TF  by leg bending stress by the ratio of section modulus or 237 MPa*(4.5/6)^3 = 100 MPa which is closer to the stress  reported by Han [4]

5.4 TF Outer Leg Bond Shear

Figure 5.4-1 TF Bending Related Bond Shear Stress

- From Dick Reed Reports/Conversations [5]:

- Shear strength, short-beam-shear, interlaminar

- Without Kapton		              65 MPa    (TF, PF1 a,b,c)

- With Kapton		       		 40 MPa (CS)

- Estimated Strength at Copper Bond   65 MPa/2 =32.5 MPa (All Coils)

- From Criteria Document:

- I-5.2.1.3  Shear Stress Allowable

- The shear-stress allowable, Ss, for an insulating material is most strongly a function of the particular material and processing method chosen, the loading conditions, the temperature, and the radiation exposure level.  The shear strength of insulating materials depends strongly on the applied compressive stress.  Therefore, the following conditions must be met for either static or fatigue conditions:

- Ss =	[2/3 to ]+ [c2 x Sc(n)]

2/3 of 32.5 MPa = 21.7 MPa

5.5 Toroidal Displacements at TF Flex Joint

Toroidal Displacements at the Flex Joint

5.6 Center Stack – TF Inner Leg Torsional Shear

Additional Discussions of torsional shear may be found in Bob Woolley’s calculation  NSTX-CALC-132-003-00 which provides moment calculations which are useful to find the maximums in thte NSTX Design Point  spreadsheet. Bob’s  summation of  the outer leg moment is directly useful in evaluations  of the up-down asymmetric case that  Han is running in the diamond truss/tangential -  radius rod calculations.

Figure 5.6-1 TF Inner Leg Torsional Shear

Figure 5.6-2 TF Inner Leg Torsional Shear

Figure 5.6-3 TF Inner Leg Torsional Shear

5.7 Outer PF Support Structure

Figure 5.7-2 6 Column Support (Danny proposes 12)

Figure 5.7-1 From an early run, the stresses at the PF4/5

support attachments to the vessel were excessive

5.8 Main Support Column to Vessel Connection

Figure 5.8-1 Bay  B-C

The main beam gusset plates are 1.5 inches thick . Visually scaling the welds, they are about 2 inches long and maybe 3/8 fillets.

Joe indicate that the weld seem to about 3/8”, definitely less than ½ “ and more than ¼ “.

He will measure to confirm.

I will ask Jim about the drawings instructions.

There are 3 on each outside edge and 3 inside- maybe more on the underside

Figure 5.8-4

Figure 5.8-3

Figure 5.8-2

Figure 5.8-5 Stresses in the vessel at the I-Beam Connection (Fully Bonded)

Figure 5.8-6 Stresses in the vessel at the I-Beam Connection (Fully Bonded)

Where the outer PF’s are supported on a separate frame, the only PF loads on the vessel result from PF1c and PF2 upper and lower. Summing these loads provides one major component of the loads that are supported by the vessel support column.

PF Coil Real Constants

Figure 5.8-7 Net Loads from the PF’s that must be Reacted by the I-eam Connection Welds

5.9 Centerstack Torsional Displacement at OH Bellevilles

Figure 5.9-1 Relative Torsional Displacements that must be allowed by the OH Belleville Precompression devices

PF Coil Real Constants

Figure 5.10-1 Net Loads on the Centerstack Assembly – See also the Monte Carlo Calculation, [10]

5.10 Net Load on Centerstack 
5.11 Vessel Shell Stresses

Vessel and umbrella structure stresses are considered in more detail in Han’s outer leg calculation [

Figure 5.11-2 Umbrella Structure Stress

Figure 5.11-1 Vessel Stresses for Normal and “Worst” Loading

4]. Note she used the vessel segment model from the global model in her analyses.
5.12 Center Stack Casing Thermal Stress

- 500C During  Plasma Operation  Ref: Art Brooks Original Calculation

- Yield of 625 at 600C is 410 MPa

- From Len’s Presentation:

- For good fatigue resistance the peak stresses in the Incoloy structure should be kept below ~380 MPa.

540MPa for 500C Plasma Operation and 400MPa for 350C Bake-Out

Figure 5.12-1

5.13 Bake-Out

Figure 5.13-1 350C Bake-Out Temp

These results were presented and the 350 C bake out temperature was questioned. It is actually 150C for the vessel and 350C for the passive plates. This analysis showed the action of the tangential radius rods allowing the growth of the vessel without disconnection of the support links.

Figure 5.13-2 Outer PF Support “Cage” is Not Connected to the Vessel During Normal Operation or Bake-Out